CODE DOCUMENTATION

System Overview: The firmware is designed as a sequential loop that gathers environmental data, logs it for post-flight analysis, and provides real-time feedback to the ground crew.

1. Initialization (Setup):

- I2C Bus Scan: At startup, the system scans addresses 0-127 to verify connections. This ensures the MPU6050 (0x68), BMP280 (0x76), and LCD (0x27) are active before launch. This also allows the system to adjust if the addresses change for unforeseen reasons.

- Hard Fault Handling: If any critical sensor or the SD card is missing during setup, the system enters a triggerFault() state, freezing execution and lighting the Red LED to alert the crew.

2. The Flight Loop:

- Data Acquisition: The MPU6050 and BMP280 are polled for acceleration, gyroscopic, temperature, and pressure data.

- Soft-Fail Logging: The logData() function attempts to write to FLIGHT.TXT. Crucially, if the SD card fails during flight (e.g., vibration), the system lights the Red LED warning but continues execution. This ensures live telemetry and LCD updates are never blocked by a storage failure.

Bugs and Self-Reflection

1.The BMP280 chip did not exist so had to use external projects to copy the specifications, However I did a typo in bmp.chip.json and wrote bm.chip.json, had to rectify the error...

2. Did not add pinMode(CS_PIN,OUTPUT) during setup, led to code failure as SD card was unable to contact Arduino

Features and Improvements:

1. Memory Optimization

- The Issue: During the integration of the SD Card module, the system initially failed to initialize the file system, even though the wiring was correct.

- Root Cause Analysis: The Arduino Uno only has 2KB of SRAM. I discovered that extensive use of Print statements (e.g., Serial.println("Scanning I2C bus...")) was filling up the dynamic memory. This left insufficient RAM for the SD library to allocate its 512-byte buffer, causing a silent failure.

- The Fix: I implemented the F() macro for all static strings (e.g., Serial.print(F("BMP OK"))). This forces the compiler to store these strings in Flash (Program) memory rather than SRAM, freeing up critical memory space for the SD card and I2C buffers to operate simultaneously.

2. Reliability

- The Issue: My initial code used a while(1) infinite loop whenever a sensor or SD card error was detected.

- Critical Flaw: While a while(1) halt is acceptable during Setup (pre-launch), using it inside the main Loop (flight) is dangerous. If the SD card disconnected for even a millisecond due to rocket vibration, the entire flight computer would freeze, killing live telemetry and the LCD display.

- The Fix: I separated error handling into two categories:

  - Hard Fail (Setup): Halts the system if critical sensors are missing before launch.

  - Soft Fail (Flight): inside logData(), if the SD write fails, the system lights the Red Fault LED but continues execution. This ensures that even if data logging fails, the ground crew still receives live telemetry via Serial and the LCD.

4. Code Modularity

- The Approach: Instead of writing a monolithic loop(), I abstracted specific hardware tasks into dedicated helper functions: scanI2C(), Display(), and logData().

- Benefit: Debugging was Easier.

Circuit Diagram